PREMIUM INDIAN CEA EQUIPMENT MANUFACTURING

R&D Strategy Report — Phase D

Bill of Materials & Raw Material / Supplier Catalog

Component-level sourcing catalog (Objective 5), prioritizing India → Taiwan/Japan/Korea → Europe/USA → China

Manufacturing base: Coimbatore, Tamil Nadu, India

Document 4 of a phased report series (Objective 5 of 6)

# Note on Scope

This is Phase D of a six-part strategic report, covering Objective 5 (Bill of Materials and Raw Material Sourcing). It provides component-level sourcing guidance for the products defined in Phase C, organized into seven functional groups rather than one undifferentiated list, for usability. Sourcing priority follows the brief's stated order — Indian suppliers first, then Taiwan/Japan/South Korea, then Europe/USA, with China used only where genuinely unavoidable — and every entry states explicitly whether that priority could be honored or not, and why.

# 1. Lighting & Optics

| Component | Material | Specification | Preferred Supplier | Country | Indian Alternative | Import Req. | MOQ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LED diodes (horticultural) | GaN-on-substrate semiconductor, 3030/2835 package | Red/blue/white mix, target 2.8-3.1 µmol/J | Seoul Semiconductor / Epistar (Ennostar) | S. Korea / Taiwan | None at required efficacy grade | Yes | 1,000-5,000 | Samsung exited LED market in 2025 — specify 'Epistar/Ennostar or equivalent' to avoid single-source risk |
| LED drivers | Constant-current/voltage switching driver | 0-10V or DALI dimmable, IP65 | MEAN WELL | Taiwan | Basic non-dimmable drivers only | Yes (India-stocked via distributors) | 50-100 | Authorized MEAN WELL distributors carry India stock, reducing effective lead time |
| Optics/reflectors | PC or PMMA lens/reflector | Beam angle 60-120°, UV-stable | Generic optics manufacturers | Taiwan/China | Basic reflectors from Indian lighting component suppliers | Partial | 500-1,000 | Only needed for spot/inter-canopy fixtures, not flat bar designs |
| Heat sinks | Extruded aluminum (6063-T5) | Custom profile per driver wattage | Cauvery Extrusions (Coimbatore) / Malabar Extrusion (Kochi) | India | This is the local option | No | Die-dependent, ~500kg+ first run | Core Coimbatore strength; new-die tooling lead time 30-60 days |
| Planted-aquarium specialty lighting | Same LED base, tuned phosphor/spectrum | Full-spectrum PAR tuned for underwater light attenuation | In-house design + local EMS assembly | India (assembly), Korea/Taiwan (diodes) | None — genuine whitespace | Partial (diodes) | Low initial volume | Differentiates from ADA (Japan) imports and generic Chinese aquascape lighting |

# 2. Structural, Enclosure & Finishing

| Component | Material | Specification | Preferred Supplier | Country | Indian Alternative | Import Req. | MOQ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Aluminum extrusion profiles | 6063-T5 aluminum alloy | Custom rack/heat-sink profiles | Cauvery Extrusions (Coimbatore) / Malabar Extrusion (Kochi) | India | Is the local option | No | Die-dependent | Plan profile designs early given die lead time |
| Stainless steel (food-contact) | SS304 sheet | 0.8-1.2mm gauge | Jindal Stainless / local Coimbatore fabricators | India | Is the local option | No | Low | SS304 minimum for food-contact microgreens trays/channels |
| Electronics/control enclosures | ABS/polycarbonate or powder-coated steel | IP54-IP65 rated | Local Coimbatore sheet-metal/injection-molding vendors | India | Is the local option | No | Low-moderate | Injection tooling cost is main constraint at low volume |
| Fasteners | Stainless/zinc-plated steel | M3-M8, DIN/ISO standard | Established Indian fastener manufacturers | India | Is the local option | No | Low | Mature, low-risk commodity category |
| Bearings | Steel/stainless ball or roller bearings | Standard metric sizes | SKF India / NBC Bearings | India (SKF has India plants) / India (NBC) | Is the local option | No | Low | For moving-rack and automation mechanisms |
| Powder coating & anodizing (finishing service) | Polyester powder / anodic oxide | RAL color-matched; QUALICOAT-grade available | Multiple qualified Coimbatore vendors | India | Is the local option | No | Per batch | Confirmed capacity per Phase C manufacturing survey |

# 3. Electronics, Control & Automation

| Component | Material | Specification | Preferred Supplier | Country | Indian Alternative | Import Req. | MOQ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Microcontrollers | ESP32 (Wi-Fi/BLE) or STM32 (ARM Cortex-M) | For sensor nodes and simple dosers | Espressif / STMicroelectronics | China (Espressif) / Europe (ST) | Board design and assembly local | Yes (chip level) | No real MOQ via distributor at low volume | Design boards in-house/Coimbatore; source chips via Mouser/Digi-Key/Indian distributors |
| Industrial PLC/HMI | Compact PLC + 7-10" touch HMI | Modbus/RS485 comms | Delta Electronics | Taiwan (India-distributed/assembled) | Larsen & Toubro and others for larger industrial scale | Partial | 1-10 units | Delta has an established India distributor/dealer network, reducing effective import friction |
| Relays | Electromechanical/solid-state relay | 5-30A, 24V/230V coil | Established Indian electrical component manufacturers | India | Is the local option | No | Low | Mature commodity |
| Touch displays | 7-10" resistive/capacitive touch HMI | Integrated with PLC or standalone | Delta / generic Chinese HMI panels | Taiwan/China | Basic Indian HMI assembly exists, limited high-quality touch options | Yes (or via Delta India) | Low |  |
| Cable harnesses | Various gauge with connectors | Custom per product | Local Coimbatore EMS/harness vendors | India | Is the local option | No | Low |  |
| Connectors (IP-rated) | M12, JST and similar | Various pin counts/ratings | Generic electronics distributors | China/Taiwan (mostly) | Limited Indian manufacture of IP-rated connectors | Yes | Low |  |
| Power supplies (SMPS) | AC-DC switching power supply | 12V/24V, various wattages | MEAN WELL | Taiwan | Indian SMPS brands adequate for non-critical loads | Partial | Low |  |

# 4. Sensors & Imaging

| Component | Material | Specification | Preferred Supplier | Country | Indian Alternative | Import Req. | MOQ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 sensor (NDIR) | Photoacoustic/NDIR CO2 module | 400-5,000 ppm, ±50ppm accuracy | Sensirion (SCD41) / DFRobot modules | Switzerland / China | None at this precision grade | Yes | Low (module-level) | Core enabling sensor for CO2 enrichment control (Phase A Sec. 1.3) |
| pH sensor | Glass electrode probe | Lab-grade, 0-14 pH, analog/BNC output | Atlas Scientific / DFRobot | USA / China | Basic Indian instrumentation probes, lower precision | Yes for lab-grade | Low |  |
| EC sensor | Conductivity probe | 0-5,000+ µS/cm | Atlas Scientific / DFRobot | USA / China | Basic Indian instrumentation probes, lower precision | Yes for lab-grade | Low |  |
| Temperature/humidity sensor | Digital sensor (SHT-series or equivalent) | ±2% RH, ±0.3°C accuracy | Sensirion | Switzerland | DHT-class sensors for non-critical use | Yes for high-precision | Low |  |
| PAR/light sensor | Quantum PAR sensor | µmol m⁻²s⁻¹ output | Apogee Instruments or generic photodiode module | USA / China | Limited | Yes | Low |  |
| Cameras (RGB/imaging) | CMOS sensor module | Fixed-mount, USB/CSI interface | Generic imaging module suppliers | China/Taiwan | Limited high-quality Indian camera module manufacture | Yes | Low |  |
| AI vision/edge compute hardware | SBC or edge-AI compute module | For imaging analytics | Raspberry Pi Foundation / NVIDIA (Jetson-class) | UK/USA | Assembled/available via Indian distributors | Partial | Low |  |

# 5. Fluid Handling — Irrigation, Fertigation & Dosing

| Component | Material | Specification | Preferred Supplier | Country | Indian Alternative | Import Req. | MOQ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Peristaltic dosing pumps | Precision low-flow pump | 0-100 ml/min typical, DC motor | Indian instrumentation suppliers (basic) / Kamoer (China) / Etatron (Italy) for precision | India (basic) / China / Italy (precision) | Partial — basic grade available locally | Partial | Low |  |
| Circulation pumps | Submersible or inline pump | Various head/flow per system | Texmo / CRI Pumps / Kirloskar Brothers | India | Is the local option | No | Low | Core Coimbatore ('Pump City of India') industry per Phase C |
| Solenoid valves | Electric solenoid valve | 12V/24V DC, various port sizes | Established Indian valve manufacturers | India | Is the local option for standard grade | No (standard) / Yes (high-precision) | Low |  |
| Filters | Inline mesh/cartridge filter | Various micron ratings | Indian filtration component suppliers | India | Is the local option | No | Low |  |
| Tubing | Food-grade silicone/PVC/PU | Various diameters | Indian tubing manufacturers | India | Is the local option | No | Low |  |
| Fertigation nutrients/fertilizers | Water-soluble NPK + micronutrient blends | Crop-specific formulations | Yara / Coromandel / Aries Agro | Europe (Yara) / India (Coromandel, Aries Agro) | Strong domestic base — preferred default | No | Low | India has a mature domestic fertilizer manufacturing sector |

# 6. Grow Media & Hydroponic Consumables

| Component | Material | Specification | Preferred Supplier | Country | Indian Alternative | Import Req. | MOQ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coco coir substrate | Processed coconut husk (coco peat) | Washed/buffered blocks, controlled EC | Vaighai Agro (Gro-Med) / Berri Coco Coir / Grow Rich India | India (Tamil Nadu/Kerala) | IS the local option — India is a global exporter | No | Low | Genuine Indian manufacturing strength; Tamil Nadu is a global coco coir hub |
| Rockwool/stone wool | Spun basalt fiber | Grodan-grade cubes/slabs | Grodan (ROCKWOOL Group) | Netherlands/Denmark | No true domestic equivalent at Grodan's quality grade | Yes | Moderate | Favor coco coir over rockwool where the crop tolerates it, to stay India-sourced |
| Perlite / vermiculite | Expanded mineral | Horticultural grade | Indian perlite/vermiculite processors | India | Is the local option | No | Low |  |
| NFT trays/channels | Food-grade plastic (injection molded) | Per product design | Sri Krishna Techno Components and similar Coimbatore molders | India | Is the local option | No | Tooling-dependent |  |
| Saffron corms | Crocus sativus planting stock | >25-30mm diameter preferred (Phase A Sec. 1.8) | Kashmir grower cooperatives (India) | India (Kashmir) | Is the local option (preferred) | No | Seasonal | Domestic Kashmir sourcing avoids import/quarantine complexity for a sterile triploid crop |

# 7. Connectivity & Cloud

| Component | Material | Specification | Preferred Supplier | Country | Indian Alternative | Import Req. | MOQ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wi-Fi/LoRa modules | Wireless connectivity module | ESP32-based Wi-Fi or long-range LoRa | Espressif / generic LoRa module makers | China | Assembled locally onto Indian-designed boards | Yes (chip level) | Low |  |
| Industrial networking gateways | RS485/Modbus-Ethernet or Modbus-TCP gateway | Protocol conversion for PLC integration | Delta / generic industrial networking suppliers | Taiwan/China | Some Indian automation integrators assemble equivalents | Partial | Low |  |
| Cloud hosting/hardware | Cloud-hosted infrastructure (service, not physical procurement) | Data logging, dashboards, alerting | AWS/Azure/Indian cloud providers | Global / India (data-residency option) | Indian data-center regions available from major providers | N/A | N/A | Recommend an India-region deployment for data residency and lower latency |

# 8. Sourcing Priority Summary

## 8.1 Genuinely India-First Categories

Aluminum extrusion and fabrication, stainless steel fabrication, fasteners, bearings, powder coating/anodizing, circulation pumps, solenoid valves (standard grade), filters, tubing, fertilizers, coco coir substrate, NFT trays, and saffron corms can all be sourced domestically today without compromising on quality — several of these (coco coir substrate and pumps in particular) are genuine Indian manufacturing export strengths, not just adequate substitutes.

## 8.2 Assemble Locally on Imported Components

LED fixtures, sensor nodes, PLC/HMI-based controllers, and dosing systems should be designed and assembled in Coimbatore using imported semiconductor/sensor/diode components (per Phase C's manufacturing feasibility conclusions) — this is standard practice even for mature global electronics brands and does not compromise the 'engineered in India' positioning, since the value-adding design and integration work stays domestic.

## 8.3 Import Initially, Revisit as Volume Grows

Precision LED diodes (Korea/Taiwan), lab-grade pH/EC/CO2/PAR sensors (USA/Switzerland/China), rockwool (Netherlands), IP-rated connectors, and camera/edge-AI modules currently have no adequate domestic source at the required grade. None of these are strategic blockers — they are standard, distributor-available components in every country's premium CEA supply chain, including the Netherlands' and Japan's own benchmark manufacturers.

## 8.4 A Genuine Cross-Cutting Risk to Flag

The LED diode market note that Samsung exited LED chip production in 2025 is a live supply-chain risk worth designing around now: fixture specifications should reference 'Epistar/Ennostar or equivalent' performance targets rather than a single named chip family, so that a future supplier exit does not force a fixture redesign.

# References

LEDYi Lighting; Fanxstar; LEDinside (2025-2026). LED chip manufacturer landscape — Seoul Semiconductor, Epistar/Ennostar, Samsung LED (2025 market exit noted).

MEAN WELL (2026). LED driver product catalog and horticulture lighting application notes.

DFRobot; Sensirion (2025-2026). NDIR/photoacoustic CO2 sensor modules (SCD41 and equivalents); pH/EC sensor product listings.

MK Flow; Kirloskar Brothers Limited; CRI Groups; Bharath Pump House (2025-2026). Top pump manufacturers in India — Kirloskar, CRI Pumps, Texmo Industries.

Delta Electronics India; deltasupplier.in; TradeIndia (2025-2026). Delta PLC/HMI India distribution network.

Grodan (ROCKWOOL Group) (2026). Dealer map and stone-wool product literature.

Vaighai Agro / Nature's Miracle (via TipRanks, 2026); Berri Coco Coir Substrate; Grow Rich India (2025-2026). Indian coco coir substrate manufacturing and export base.

IndiaMART / TradeIndia directories (2025-2026). Coimbatore aluminum extrusion, anodizing, powder coating, fastener, and bearing supplier listings; Sri Krishna Techno Components (injection molding/die casting).

Phase A, B & C of this report series (internal cross-reference) — subsystem architecture, global benchmark, and product portfolio underpinning the sourcing decisions above.